# Wi-SUN Corridor Streetlight - ZG28 LED Control System

## Project Overview

This project implements a Wi-SUN FAN (Field Area Network) based smart streetlight control system using the Silicon Labs EFR32ZG28B312F1024IM48 wireless SoC on the BRD2705A (ZG28) development board. The system provides remote LED/Relay control via CoAP protocol over Wi-SUN mesh network, making it ideal for smart city streetlight applications with centralized monitoring and control.

## Hardware Configuration

### Main Components

- MCU: EFR32ZG28B312F1024IM48 (ZG28)

  - ARM Cortex-M33 core

  - 1024 KB Flash, 256 KB RAM

  - Sub-GHz radio with up to +20 dBm output power

- Development Board: BRD2705A (ZG28 Radio Board)

  - Integrated antenna for Sub-GHz operation

  - USB interface for programming and debugging

  - UART console via VCOM

- Radio Configuration:

  - Sub-GHz Wi-SUN FAN 1.0/1.1 and HAN compliant

  - Frequency hopping spread spectrum (FHSS)

  - Supports regional domains (FCC, ETSI, etc.)

### LED/Relay Pin Configuration

The system uses 5 GPIO pins from Port C for controlling LEDs or external relays:

| Component | GPIO Port | GPIO Pin | Physical Pin | Function | Active State |
| --- | --- | --- | --- | --- | --- |
| LED0 / RELAY1 | Port C | Pin 1 | PC1 | Streetlight Control 1 | Active HIGH |
| LED1 / RELAY2 | Port C | Pin 2 | PC2 | Streetlight Control 2 | Active HIGH |
| LED2 / RELAY3 | Port C | Pin 3 | PC3 | Streetlight Control 3 | Active HIGH |
| LED3 / RELAY4 | Port C | Pin 4 | PC4 | Streetlight Control 4 | Active HIGH |
| LED4 | Port C | Pin 9 | PC9 | Wi-SUN Connection Status | Active HIGH |

Note: LED0-LED3 are mapped as RELAY1-RELAY4 in the software for streetlight control applications.

### Hardware Connection Diagram

┌─────────────────────────────────────────────────────────────┐
│           EFR32ZG28B312 (BRD2705A Board)                    │
│                                                               │
│   ┌──────────────────────────────────────────────┐          │
│   │                                                │          │
│   │     PC1 (LED0/RELAY1) ──────────┬────► Relay Module 1   │
│   │                                  │                       │
│   │     PC2 (LED1/RELAY2) ──────────┼────► Relay Module 2   │
│   │                                  │                       │
│   │     PC3 (LED2/RELAY3) ──────────┼────► Relay Module 3   │
│   │                                  │                       │
│   │     PC4 (LED3/RELAY4) ──────────┼────► Relay Module 4   │
│   │                                  │                       │
│   │     PC9 (LED4) ─────────────────┴────► Status LED       │
│   │                                                │          │
│   │                                                │          │
│   │     Sub-GHz Radio ◄──────► Wi-SUN Network    │          │
│   │                                                │          │
│   │     UART/USB ◄──────────► PC (Console/Debug) │          │
│   │                                                │          │
│   └──────────────────────────────────────────────┘          │
│                                                               │
└─────────────────────────────────────────────────────────────┘

External Connections:
┌──────────────┐      ┌──────────────┐      ┌──────────────┐
│   Relay      │      │  Streetlight │      │   Power      │
│   Module     │─────►│   Driver     │─────►│   Supply     │─────► AC Lights
│   (PC1-PC4)  │      │   Circuit    │      │   (AC/DC)    │
└──────────────┘      └──────────────┘      └──────────────┘

### Relay Module Interface

For controlling AC streetlights, connect GPIO pins to relay modules:

- Recommended: 5V relay modules with optocoupler isolation

- Current Drive: Use transistor driver if GPIO current is insufficient

- Protection: Add flyback diodes for relay coil protection

- Isolation: Optocoupler isolation between MCU and high-voltage circuits

Example Connection (Single Relay):

EFR32ZG28                 Relay Module              AC Load
┌─────────┐              ┌──────────┐           ┌──────────┐
│         │              │          │           │          │
│  PC1 ───┼──► VCC      │ Relay    │           │  Street  │
│         │              │ Coil     ├───────────┤  Light   │
│  GND ───┼──► GND      │          │           │          │
│         │              │          │           │          │
│         │              │  NO/NC   │           │          │
└─────────┘              └──────────┘           └──────────┘

### Power Supply

- USB Power: 5V via USB (development/testing)

- External Power: 3.3V regulated supply for production deployment

- Relay Power: Separate 5V/12V supply for relay modules (isolated from MCU)

## Key Features

### 1. Wi-SUN Mesh Network Connectivity

- Full Wi-SUN FAN 1.0, FAN 1.1, and HAN profile support

- Automatic network joining and parent discovery

- Self-healing mesh topology

- Long-range Sub-GHz communication

- IPv6 addressing with DHCP

### 2. Remote LED/Relay Control via CoAP

The system provides 8 CoAP endpoints for controlling 4 independent streetlight circuits:

#### Relay Control Endpoints

| CoAP URI | Method | Function | GPIO Pin | Response |
| --- | --- | --- | --- | --- |
| /relay1on | GET/POST | Turn ON Relay 1 | PC1 | {"Message": "RELAY1 ON!"} |
| /relay1off | GET/POST | Turn OFF Relay 1 | PC1 | {"Message": "RELAY1 OFF!"} |
| /relay2on | GET/POST | Turn ON Relay 2 | PC2 | {"Message": "RELAY2 ON!"} |
| /relay2off | GET/POST | Turn OFF Relay 2 | PC2 | {"Message": "RELAY2 OFF!"} |
| /relay3on | GET/POST | Turn ON Relay 3 | PC3 | {"Message": "RELAY3 ON!"} |
| /relay3off | GET/POST | Turn OFF Relay 3 | PC3 | {"Message": "RELAY3 OFF!"} |
| /relay4on | GET/POST | Turn ON Relay 4 | PC4 | {"Message": "RELAY4 ON!"} |
| /relay4off | GET/POST | Turn OFF Relay 4 | PC4 | {"Message": "RELAY4 OFF!"} |

Usage Example:

# Turn on streetlight 1
coap-client -m get coap://[device-ipv6]:5683/relay1on

# Turn off streetlight 2
coap-client -m get coap://[device-ipv6]:5683/relay2off

# Control all lights
coap-client -m get coap://[device-ipv6]:5683/relay1on
coap-client -m get coap://[device-ipv6]:5683/relay2on
coap-client -m get coap://[device-ipv6]:5683/relay3on
coap-client -m get coap://[device-ipv6]:5683/relay4on

### 3. Network Status Indicator

- LED4 (PC9): Automatically turns ON when device successfully connects to Wi-SUN network

- Provides instant visual feedback of network connectivity

- OFF state indicates disconnection or joining in progress

### 4. Initial State Configuration

- All relays initialize to ON state at startup

- Can be modified in app_coap_resources_init() function

- Ensures known state on power-up/reset

## Software Architecture

### Core Modules

- main.c - Application entry point and system initialization

- app.c/h - Main application logic and Wi-SUN event handling

- app_init.c/h - Application initialization routines

- app_coap.c/h - CoAP server with relay control handlers

- app_check_neighbors.c/h - Network neighbor management

- app_timestamp.c/h - Timestamp utilities for logging

- app_rtt_traces.c/h - RTT debug output utilities

### LED/Relay Control Implementation

The relay control is implemented in app_coap.c with the following key functions:

// Relay 1 Control (LED0 / PC1)
coap_callback_relay1on()   // Turn ON relay 1
coap_callback_relay1off()  // Turn OFF relay 1

// Relay 2 Control (LED1 / PC2)
coap_callback_relay2on()   // Turn ON relay 2
coap_callback_relay2off()  // Turn OFF relay 2

// Relay 3 Control (LED2 / PC3)
coap_callback_relay3on()   // Turn ON relay 3
coap_callback_relay3off()  // Turn OFF relay 3

// Relay 4 Control (LED3 / PC4)
coap_callback_relay4on()   // Turn ON relay 4
coap_callback_relay4off()  // Turn OFF relay 4

### GPIO Control Functions

sl_simple_led_turn_on(sl_led_led0.context);   // GPIO HIGH
sl_simple_led_turn_off(sl_led_led0.context);  // GPIO LOW
sl_simple_led_get_state(sl_led_led0.context); // Read current state

### Dependencies

- Silicon Labs Simplicity SDK: 2025.6.2

- RTOS: Micrium OS Kernel

- Network Stack: Wi-SUN Stack with CoAP support

- Security: mbedTLS with PSA Crypto

- Storage: NVM3 (Non-Volatile Memory)

- Console: iostream with UART/EUSART VCOM

## Configuration

### Key Configuration Options

// CoAP Configuration
SL_WISUN_COAP_RESOURCE_HND_MAX_RESOURCES = 30U

// OTA DFU
SL_WISUN_OTA_DFU_AUTO_INSTALL_ENABLED = 0U

// UART Console
SL_BOARD_ENABLE_VCOM = 1
SL_IOSTREAM_EUSART_VCOM_CONVERT_BY_DEFAULT_LF_TO_CRLF = 1

// PTI (Packet Trace Interface)
SL_RAIL_UTIL_PTI_BAUD_RATE_HZ = 3200000

### Radio Configuration

- Profile: Single PHY Wi-SUN

- Supported Profiles: FAN 1.0, FAN 1.1, HAN

- PA Selection: Sub-GHz with EFF support (if available)

- Max TX Power: 20 dBm (with EFF)

- Duty Cycle: 100%

## Building the Project

### Prerequisites

- Simplicity Studio 5 (SV5)

- Simplicity SDK 2025.6.2 or later

- GNU ARM Toolchain v12.2.1 or compatible

- SEGGER J-Link debugger (integrated with BRD2705A)

### Build Steps

- Open Simplicity Studio 5

- Import the project using the .slcp file

- Select the target device: EFR32ZG28B312F1024IM48

- Select the target board: BRD2705A

- Build the project: Project → Build Project

- Flash to device: Run → Debug or use Flash Programmer

### Configuration Files

- config/sl_simple_led_led0_config.h - LED0/Relay1 GPIO configuration

- config/sl_simple_led_led1_config.h - LED1/Relay2 GPIO configuration

- config/sl_simple_led_led2_config.h - LED2/Relay3 GPIO configuration

- config/sl_simple_led_led3_config.h - LED3/Relay4 GPIO configuration

- config/sl_simple_led_led4_config.h - LED4 status indicator configuration

## Usage

### Initial Setup

- Connect the ZG28 board to your PC via USB

- Flash the firmware to the device

- Open a serial terminal (115200 baud, 8N1) to view console output

- The device will automatically attempt to join the Wi-SUN network

- Once connected, LED4 will turn ON

- All relays (LED0-LED3) will initialize to ON state

### Console Output

The application provides detailed logging:

Wi-SUN network joining...
Join state: SELECT_PAN
Join state: AUTHENTICATE
Join state: ACQUIRE_PAN_CONFIG
Join state: CONFIGURE_ROUTING
Join state: OPERATIONAL
[Connected to Wi-SUN network]
Device IPv6: fd00:7283:7e00:0:202:f7ff:fe00:xxxx
30/30 CoAP resources added to CoAP Resource handler

### Controlling Streetlights via CoAP

#### Using libcoap Command-Line Client

# Get device IPv6 address from console output
DEVICE_IP="fd00:7283:7e00:0:202:f7ff:fe00:xxxx"

# Control individual streetlights
coap-client -m get "coap://[$DEVICE_IP]:5683/relay1on"
coap-client -m get "coap://[$DEVICE_IP]:5683/relay1off"

coap-client -m get "coap://[$DEVICE_IP]:5683/relay2on"
coap-client -m get "coap://[$DEVICE_IP]:5683/relay2off"

coap-client -m get "coap://[$DEVICE_IP]:5683/relay3on"
coap-client -m get "coap://[$DEVICE_IP]:5683/relay3off"

coap-client -m get "coap://[$DEVICE_IP]:5683/relay4on"
coap-client -m get "coap://[$DEVICE_IP]:5683/relay4off"

#### Using Python CoAP Client

from aiocoap import *

async def control_streetlight(ipv6_address, relay_num, state):
    """
    Control streetlight relay
    relay_num: 1-4
    state: 'on' or 'off'
    """
    uri = f'coap://[{ipv6_address}]:5683/relay{relay_num}{state}'
    protocol = await Context.create_client_context()
    request = Message(code=GET, uri=uri)
    
    try:
        response = await protocol.request(request).response
        print(f'Response: {response.payload.decode()}')
    except Exception as e:
        print(f'Failed: {e}')

# Example usage
import asyncio
asyncio.run(control_streetlight('fd00:7283:7e00:0:202:f7ff:fe00:xxxx', 1, 'on'))

#### Using Web-Based CoAP Client (Copper)

- Install Copper (Cu) CoAP browser extension

- Enter CoAP URI: coap://[device-ipv6]:5683/

- Discover available resources

- Click on /relay1on, /relay1off, etc. to control

### LED Indicators During Operation

- LED4 ON: Successfully connected to Wi-SUN network

- LED4 OFF: Disconnected or joining network

- LED0-LED3: Reflect relay states (ON/OFF based on CoAP commands)

## Network Integration

### Wi-SUN Network Requirements

- Border Router: A Wi-SUN border router must be configured and operational

- Network Parameters: Match network name, regulatory domain, and security settings

- IPv6 Configuration: The device obtains a global IPv6 address via DHCP

- Firewall: Ensure CoAP port 5683 is accessible from control center

### Network Topology for Streetlight System

Internet/Cloud
                         ↕
              ┌──────────────────┐
              │  Border Router   │
              │   (Root Node)    │
              └──────────────────┘
                       ↕
           ┌───────────┴───────────┐
           ↕                       ↕
    ┌─────────────┐         ┌─────────────┐
    │  Router 1   │←─────→  │  Router 2   │
    │ (Repeater)  │         │ (Repeater)  │
    └─────────────┘         └─────────────┘
           ↕                       ↕
      ┌────┴────┐             ┌────┴────┐
      ↕         ↕             ↕         ↕
  ┌───────┐ ┌───────┐    ┌───────┐ ┌───────┐
  │Street │ │Street │    │Street │ │Street │
  │Light1 │ │Light2 │    │Light3 │ │Light4 │
  │(This) │ │       │    │       │ │       │
  └───────┘ └───────┘    └───────┘ └───────┘

### Control Center Integration

For centralized streetlight management, integrate with:

- SCADA Systems: Use CoAP-to-MQTT bridge

- IoT Platforms: AWS IoT, Azure IoT, Google Cloud IoT

- Custom Dashboard: Web interface with CoAP client library

## Customization

### Modifying Initial Relay States

Edit app_coap.c in the app_coap_resources_init() function:

// Current: All relays ON at startup
sl_simple_led_turn_on(sl_led_led0.context);  // Relay 1 ON
sl_simple_led_turn_on(sl_led_led1.context);  // Relay 2 ON
sl_simple_led_turn_on(sl_led_led2.context);  // Relay 3 ON
sl_simple_led_turn_on(sl_led_led3.context);  // Relay 4 ON

// Change to: All relays OFF at startup
sl_simple_led_turn_off(sl_led_led0.context);  // Relay 1 OFF
sl_simple_led_turn_off(sl_led_led1.context);  // Relay 2 OFF
sl_simple_led_turn_off(sl_led_led2.context);  // Relay 3 OFF
sl_simple_led_turn_off(sl_led_led3.context);  // Relay 4 OFF

### Adding Relay State Query Endpoints

Uncomment the following sections in app_coap.c to enable state query:

// Uncomment these blocks (around line 1073-1104)
coap_resource.data.uri_path = "/relay1state";
coap_resource.auto_response = coap_callback_relay1state;
// ... (repeat for relay2state, relay3state, relay4state)

This enables:

- /relay1state - Returns current state of Relay 1

- /relay2state - Returns current state of Relay 2

- /relay3state - Returns current state of Relay 3

- /relay4state - Returns current state of Relay 4

### Changing GPIO Pins

To use different GPIO pins, modify the config files:

File: config/sl_simple_led_led0_config.h

#define SL_SIMPLE_LED_LED0_PORT    SL_GPIO_PORT_C  // Change port
#define SL_SIMPLE_LED_LED0_PIN     1               // Change pin number

Repeat for LED1-LED4 configuration files.

### Adding Scheduling Logic

Implement time-based control by adding a timer:

// In app.c - Add timer callback
void schedule_callback(void) {
    uint32_t current_hour = get_current_hour(); // Implement time sync
    
    if (current_hour >= 18 || current_hour < 6) {
        // Turn on lights at night (6 PM - 6 AM)
        sl_simple_led_turn_on(sl_led_led0.context);
        sl_simple_led_turn_on(sl_led_led1.context);
    } else {
        // Turn off lights during day
        sl_simple_led_turn_off(sl_led_led0.context);
        sl_simple_led_turn_off(sl_led_led1.context);
    }
}

## Troubleshooting

### Device Won’t Join Network

- Check Border Router: Verify border router is operational and broadcasting

- Network Credentials: Ensure network name and security keys match

- Regulatory Domain: Verify frequency band settings match region

- Antenna: Check antenna connection on BRD2705A board

- Console Logs: Review join state transitions for specific errors

### CoAP Commands Not Working

- IPv6 Address: Verify device IPv6 address from console output

- Network Reachability: Ping the device: ping6 [device-ipv6]

- Firewall: Check firewall rules allow UDP port 5683

- CoAP Client: Test with different clients (libcoap, Copper, Python)

- Syntax: Ensure proper IPv6 format: coap://[ipv6]:5683/relay1on

### LED4 Not Turning On

- Connection Status: Check console for “OPERATIONAL” join state

- GPIO Configuration: Verify PC9 configuration in config files

- Code Check: Ensure LED4 control code in app.c is not commented out

### Relays Not Responding

- GPIO Pins: Verify PC1-PC4 are correctly configured

- Hardware: Check relay module connections and power supply

- Driver Circuit: Ensure sufficient current drive for relay coils

- Test LED: Connect LED instead of relay to verify GPIO toggling

### Console Output Issues

- Baud Rate: Set terminal to 115200, 8N1, no flow control

- USB Driver: Install Silicon Labs USB-to-UART driver

- Port Selection: Use correct COM port for BRD2705A VCOM

## Performance Specifications

### Network Performance

- Range: Up to 1-2 km line-of-sight (Sub-GHz)

- Data Rate: 50-300 kbps (depends on PHY configuration)

- Latency: 100-500 ms typical for CoAP command response

- Network Capacity: Supports 100+ nodes per network

### Power Consumption

- Active (TX at 20 dBm): ~100-150 mA

- Active (RX): ~30-50 mA

- Idle (Network Connected): ~10-20 mA

- Deep Sleep: <10 µA (requires code modification)

### Relay Specifications

- GPIO Output: 3.3V logic level

- Drive Current: Limited by GPIO (8-10 mA typical)

- Recommendation: Use relay module with driver transistor

- Switching Frequency: Not intended for PWM (DC control only)

## Application Scenarios

### Street Lighting Control

- Deploy multiple nodes along streets

- Central control via CoAP commands

- Schedule-based automation

- Energy consumption monitoring

### Parking Lot Lighting

- Zone-based control (4 relays per node)

- Motion sensor integration possible

- Fault detection and reporting

### Industrial Lighting

- Factory/warehouse lighting control

- Integration with building management systems

- Emergency lighting override capability

### Smart City Integration

- Part of larger IoT infrastructure

- Integration with traffic management

- Adaptive lighting based on conditions

## Development Notes

### Code Quality

This is EXPERIMENTAL QUALITY code:

- Not formally tested for production environments

- Provided as-is for development and evaluation purposes

- Use as reference for building production applications

- Requires additional testing and hardening for commercial deployment

### Memory Configuration

- Flash Usage: ~500-600 KB (depends on configuration)

- RAM Usage: ~100-150 KB (stack + heap)

- NVM3: Default configuration for persistent storage

- CoAP Resources: Maximum 30 resources configured

### Important Implementation Notes

- Relay Control Logic: Uses inverted logic (turn_off = ON, turn_on = OFF) due to hardware design

- Active HIGH: All LEDs configured for active HIGH operation

- Blocking Operations: CoAP handlers execute in callback context

- Thread Safety: Use RTOS primitives for shared resource access

## Security Considerations

### Network Security

- Wi-SUN uses IEEE 802.15.4 security (AES-128)

- Certificate-based authentication supported

- Network key management via border router

- Secure key storage in flash memory

### Application Security

- CoAP over DTLS not implemented (can be added)

- No authentication on CoAP endpoints (open access)

- Recommendation: Implement CoAP token verification

- Use network-level security (firewall, VPN)

### Secure Deployment Checklist

- [ ] Change default network credentials

- [ ] Enable CoAP DTLS if available

- [ ] Implement access control lists

- [ ] Regular firmware updates via OTA

- [ ] Monitor for unauthorized access attempts

- [ ] Physical security of hardware

## License

SPDX-License-Identifier: Zlib

Copyright 2023 Silicon Laboratories Inc. www.silabs.com

This software is provided ‘as-is’, without any express or implied warranty. See source files for complete license text.

## Bill of Materials (BOM)

### Development Setup

| Item | Part Number | Quantity | Purpose |
| --- | --- | --- | --- |
| ZG28 Radio Board | BRD2705A | 1 | Main MCU board |
| USB Cable | Micro USB | 1 | Programming & power |
| 5V Relay Module | Generic 4-channel | 1 | Streetlight switching |
| Jumper Wires | Female-Female | 10+ | GPIO connections |

### Production Deployment (per node)

| Item | Specification | Quantity | Purpose |
| --- | --- | --- | --- |
| EFR32ZG28 Module | Custom or certified module | 1 | Wireless SoC |
| Relay Module | 5V/12V 4-channel optocoupled | 1 | Load switching |
| Power Supply | 5V/3.3V regulated | 1 | System power |
| Antenna | Sub-GHz external | 1 | RF transmission |
| Enclosure | IP65 rated weatherproof | 1 | Protection |
| Connectors | Phoenix/terminal blocks | 1 set | Wiring |

## References

- Wi-SUN Alliance

- Silicon Labs Wi-SUN Documentation

- EFR32ZG28 Product Page

- BRD2705A User Guide

- Simplicity Studio 5

- CoAP RFC 7252

- libcoap Library

## Support

For technical support and questions:

- Silicon Labs Community Forum

- Silicon Labs Support Portal

- GitHub Issues (if applicable)

Project: scrc_streetlight
Version: 1.0.0
SDK Version: Simplicity SDK 2025.6.2
Board: BRD2705A (ZG28)
Chip: EFR32ZG28B312F1024IM48
Category: Wi-SUN Smart Streetlight Control
Last Updated: January 2026